# Student Quick-Reference Guide: Steelwork Design

# 1. Material Properties & Basics

- Young's Modulus (E): 205,000  N/mm2

- Steel Grade Constant (ε): ε = (275py)0.5

- Design Strength (py): Varies by steel grade and plate thickness (e.g., for S355 and thickness <= 16 mm, py = 355 N/mm2).

# 2. Cross-Section Capacity (Member Checks)

- Bending Moment Capacity:

- Mcx= py Sx <= 1.2 py Zx

- Mcy= py Sy <= 1.2 py Zy

- Shear Capacity:

- Vc = py Av3 (where Av = tD for rolled I, H, and channel sections)

- Tension Capacity:

- Pt = Ae py (with net area adjustments depending on connection types like bolts or welds)

- Compression Capacity:

- Pc = Ag pc (where slenderness λ = Le / r)

# 3. Deflection Limits

- Maximum Deflection Formulas:

- Uniformly Distributed Load (UDL): Δ = 5384wl4EI

- Mid-span Point Load: Δ = Pl348EI

- Common Deflection Limits (Table 5.1):

- Cantilevers: Length/ 180

- Beams carrying plaster/brittle finishes: Span/ 360

- Other beams: Span/ 200

# 4. Connections & Bolts

- Bolt Shear Capacity: Ps = ps As

- Bolt Tension Capacity: Pt = As pt (or Pnom= 0.8 As pt)

- Bolt Bearing Capacity: Pbb= d tp pbb

- Fillet Weld Strength: Strength= 0.7 × (leg length) × pw × 10-3 kN/mm